/******************************************************

* ESP32 + MPU6050 (0x69) + OLED + WiFi + ThingSpeak

* Displays ACC & GYRO, uploads to cloud

******************************************************/

#include <Wire.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_SSD1306.h>

#include <Adafruit_GFX.h>

/* ---------- OLED ---------- */

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

#define OLED_ADDR 0x3C

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

/* ---------- MPU6050 ---------- */

Adafruit_MPU6050 mpu;

/* ---------- WiFi ---------- */

const char* ssid     = "Shreeeeee";

const char* password = "12345678";

/* ---------- ThingSpeak ---------- */

String apiKey = "YOUR_THINGSPEAK_API_KEY";

/* ---------- Timing ---------- */

unsigned long lastUpload = 0;

const unsigned long uploadInterval = 20000; // ThingSpeak minimum = 15s

void setup() {

Serial.begin(115200);

Wire.begin(21, 22);

/* OLED */

if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("OLED not found");

while (1);

}

/* MPU6050 @ 0x69 */

if (!mpu.begin(0x69)) {

Serial.println("MPU6050 not found");

while (1);

}

mpu.setAccelerometerRange(MPU6050_RANGE_8_G);

mpu.setGyroRange(MPU6050_RANGE_500_DEG);

mpu.setFilterBandwidth(MPU6050_BAND_21_HZ);

/* Initial display */

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(10, 25);

display.println("ACC & GYRO LIVE");

display.display();

delay(1500);

/* WiFi */

connectWiFi();

}

void loop() {

sensors_event_t acc, gyro, temp;

mpu.getEvent(&acc, &gyro, &temp);

/* -------- OLED DISPLAY (FIRST PRIORITY) -------- */

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("AX AY AZ | GX GY GZ");

display.setCursor(0, 14);

display.print("AX:");

display.print(acc.acceleration.x, 1);

display.setCursor(64, 14);

display.print("GX:");

display.print(gyro.gyro.x, 1);

display.setCursor(0, 26);

display.print("AY:");

display.print(acc.acceleration.y, 1);

display.setCursor(64, 26);

display.print("GY:");

display.print(gyro.gyro.y, 1);

display.setCursor(0, 38);

display.print("AZ:");

display.print(acc.acceleration.z, 1);

display.setCursor(64, 38);

display.print("GZ:");

display.print(gyro.gyro.z, 1);

display.setCursor(0, 52);

display.print("WiFi:");

display.print(WiFi.status() == WL_CONNECTED ? "OK" : "NO");

display.display();

/* -------- ThingSpeak Upload -------- */

if (WiFi.status() == WL_CONNECTED &&

millis() - lastUpload > uploadInterval) {

sendToThingSpeak(acc, gyro);

lastUpload = millis();

}

delay(50);  // smooth display

}

/* ================= FUNCTIONS ================= */

void connectWiFi() {

display.clearDisplay();

display.setCursor(0, 25);

display.println("Connecting WiFi");

display.display();

WiFi.begin(ssid, password);

int tries = 0;

while (WiFi.status() != WL_CONNECTED && tries < 20) {

delay(500);

tries++;

}

display.clearDisplay();

display.setCursor(0, 25);

display.println(WiFi.status() == WL_CONNECTED ?

"WiFi Connected" : "WiFi Failed");

display.display();

delay(1000);

}

void sendToThingSpeak(sensors_event_t acc, sensors_event_t gyro) {

HTTPClient http;

String url = "http://api.thingspeak.com/update?api_key=" + apiKey +

"&field1=" + String(acc.acceleration.x, 2) +

"&field2=" + String(acc.acceleration.y, 2) +

"&field3=" + String(acc.acceleration.z, 2) +

"&field4=" + String(gyro.gyro.x, 2) +

"&field5=" + String(gyro.gyro.y, 2) +

"&field6=" + String(gyro.gyro.z, 2);

http.begin(url);

int httpCode = http.GET();

http.end();

Serial.println("ThingSpeak upload: " + String(httpCode));

}

Last working code